# SOURCE CODE

An Integrated Smart Helmet System Using IoT for Enhanced Rider Safety

# 1. ESP32 Working Code (Basic Test Program)

#define LED 2

void setup() {

Serial.begin(115200);

pinMode(LED, OUTPUT);

Serial.println("ESP32 Working Test Started");

}

void loop() {

digitalWrite(LED, HIGH);

Serial.println("LED ON");

delay(1000);

digitalWrite(LED, LOW);

Serial.println("LED OFF");

delay(1000);

}

# 2. GPS Working Code

#include <WiFi.h>

#include <WiFiClientSecure.h>

#include <UniversalTelegramBot.h>

#include <TinyGPS++.h>

#include <HardwareSerial.h>

// WIFI

const char* ssid = "iQOO Neo9 Pro";

const char* password = "00000000";

// TELEGRAM

#define BOTtoken "8779350127:AAGcdWMyOReix_2yEzNSydrm4vIbwMgM20g"

#define CHAT_ID "7912848395"

WiFiClientSecure client;

UniversalTelegramBot bot(BOTtoken, client);

// GPS

TinyGPSPlus gps;

HardwareSerial gpsSerial(2);

// GPS Pins

#define GPS_RX 16

#define GPS_TX 17

unsigned long lastSend = 0;

void setup() {

Serial.begin(115200);

// Start GPS

gpsSerial.begin(9600, SERIAL_8N1, GPS_RX, GPS_TX);

// Connect WiFi

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi Connected!");

client.setInsecure();

bot.sendMessage(CHAT_ID, "GPS Tracking System Started", "");

}

void loop() {

// Read GPS data

while (gpsSerial.available() > 0) {

gps.encode(gpsSerial.read());

}

// Send every 10 sec

if (millis() - lastSend > 10000) {

String message = "";

if (gps.location.isValid()) {

float lat = gps.location.lat();

float lng = gps.location.lng();

Serial.print("Lat: "); Serial.println(lat);

Serial.print("Lng: "); Serial.println(lng);

message += "Live Location:\n";

message += "https://maps.google.com/?q=";

message += String(lat, 6);

message += ",";

message += String(lng, 6);

} else {

Serial.println("GPS NOT FIXED");

message = "GPS: Not Fixed (Go outside)";

}

// Send to Telegram

bot.sendMessage(CHAT_ID, message, "");

lastSend = millis();

}

}

# 3. Helmet Side Code (Transmitter)

#include <WiFi.h>

#include <esp_now.h>

#include <Wire.h>

#define LIMIT_SWITCH 4

#define MQ3_PIN 34

#define BUZZER 26

#define LED 2

#define MPU_ADDR 0x68

uint8_t bikeMAC[] = {0x28,0x05,0xA5,0x34,0xD5,0x7C};

typedef struct {

bool helmet;

bool alcohol;

bool accident;

int mqValue;

float accTotal;

} Data;

Data data;

bool helmet=false, alcohol=false, accident=false;

int mq=0;

int16_t ax, ay, az;

float total=0;

// Send status

void onSent(const wifi_tx_info_t *info, esp_now_send_status_t status) {

Serial.print("Send: ");

Serial.println(status == ESP_NOW_SEND_SUCCESS ? "OK" : "FAIL");

}

void setup() {

Serial.begin(115200);

pinMode(LIMIT_SWITCH, INPUT_PULLUP);

pinMode(BUZZER, OUTPUT);

pinMode(LED, OUTPUT);

// I2C

Wire.begin(21,22);

// Wake MPU (important)

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x6B);

Wire.write(0);

Wire.endTransmission(true);

// ESP-NOW Setup

WiFi.mode(WIFI_STA);

if (esp_now_init() != ESP_OK) {

Serial.println("ESP-NOW Init Failed");

return;

}

esp_now_register_send_cb(onSent);

esp_now_peer_info_t peerInfo = {};

memcpy(peerInfo.peer_addr, bikeMAC, 6);

peerInfo.channel = 0;

peerInfo.encrypt = false;

if (esp_now_add_peer(&peerInfo) != ESP_OK) {

Serial.println("Peer Add Failed");

return;

}

Serial.println("=== HELMET SYSTEM READY ===");

}

void loop() {

// Helmet detection

helmet = (digitalRead(LIMIT_SWITCH)==LOW);

// MQ3 reading

mq = analogRead(MQ3_PIN);

alcohol = (mq > 1800);

// MPU reading (safe)

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x3B);

if (Wire.endTransmission(false) == 0 && Wire.requestFrom(MPU_ADDR, 6, true) == 6) {

ax = Wire.read()<<8 | Wire.read();

ay = Wire.read()<<8 | Wire.read();

az = Wire.read()<<8 | Wire.read();

total = sqrt((float)ax*ax + (float)ay*ay + (float)az*az);

accident = (total > 20000);

} else {

// If read fails -> avoid false trigger

accident = false;

total = 0;

}

// PRIORITY LOGIC

if (!helmet) {

alcohol = false;

accident = false;

digitalWrite(BUZZER, LOW);

digitalWrite(LED, LOW);

}

else if (accident) {

digitalWrite(BUZZER, HIGH);

digitalWrite(LED, HIGH);

}

else if (alcohol) {

digitalWrite(LED, millis()%400 < 200); // blink

digitalWrite(BUZZER, LOW);

}

else {

digitalWrite(BUZZER, LOW);

digitalWrite(LED, LOW);

}

// Send data

data.helmet = helmet;

data.alcohol = alcohol;

data.accident = accident;

data.mqValue = mq;

data.accTotal = total;

esp_now_send(bikeMAC,(uint8_t*)&data,sizeof(data));

// Clean Serial Output

Serial.print("H:"); Serial.print(helmet);

Serial.print(" MQ:"); Serial.print(mq);

Serial.print(" ALC:"); Serial.print(alcohol);

Serial.print(" ACC:"); Serial.print(accident);

Serial.print(" T:"); Serial.println(total);

delay(300);

}

# 4. Bike Side Code (Receiver)

#define BLYNK_TEMPLATE_ID "TMPL31mQ4Aend"

#define BLYNK_TEMPLATE_NAME "Smart Helmet"

#define BLYNK_AUTH_TOKEN "PhnfhMZ5giDcBKoa9sazDrpIQUzPEN5_"

#define BLYNK_PRINT Serial

#include <WiFi.h>

#include <esp_now.h>

#include <BlynkSimpleEsp32.h>

#include <WiFiClientSecure.h>

// WiFi

char ssid[] = "iQOO Neo9 Pro";

char pass[] = "00000000";

// Telegram

String botToken = "8779350127:AAGcdWMyOReix_2yEzNSydrm4vIbwMgM20g";

String chatID = "7912848395";

WiFiClientSecure client;

// OUTPUTS

#define RELAY 27

#define RED 26

#define BLUE 2

#define BUZZER 14

// Data structure

typedef struct {

bool helmet;

bool alcohol;

bool accident;

int mqValue;

float accTotal;

} Data;

Data incoming = {false,false,false,0,0};

// Receive data

void onReceive(const esp_now_recv_info_t *info, const uint8_t *data, int len) {

memcpy(&incoming, data, sizeof(incoming));

}

// Telegram

void sendTelegram(String msg) {

client.setInsecure();

String url = "/bot" + botToken + "/sendMessage?chat_id=" + chatID + "&text=" + msg;

if (client.connect("api.telegram.org", 443)) {

client.print(String("GET ") + url + " HTTP/1.1\r\nHost: api.telegram.org\r\nConnection: close\r\n\r\n");

}

}

// Alert flag

bool alertSent = false;

void setup() {

Serial.begin(115200);

pinMode(RELAY, OUTPUT);

pinMode(RED, OUTPUT);

pinMode(BLUE, OUTPUT);

pinMode(BUZZER, OUTPUT);

digitalWrite(RELAY, HIGH); // Bike OFF initially

// WiFi + Blynk

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, pass);

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

// ESP-NOW

if (esp_now_init() != ESP_OK) {

Serial.println("ESP-NOW Init Failed");

return;

}

esp_now_register_recv_cb(onReceive);

Serial.println("=== BIKE SYSTEM READY ===");

}

void loop() {

Blynk.run();

Serial.println("===== BIKE STATUS =====");

Serial.print("Helmet: "); Serial.println(incoming.helmet);

Serial.print("Alcohol: "); Serial.println(incoming.alcohol);

Serial.print("Accident: "); Serial.println(incoming.accident);

// PRIORITY LOGIC

if (!incoming.helmet) {

// Helmet not worn

digitalWrite(RELAY, HIGH);

digitalWrite(RED, HIGH);

digitalWrite(BLUE, LOW);

digitalWrite(BUZZER, LOW);

Serial.println("HELMET NOT WORN - BIKE OFF");

alertSent = false;

}

else if (incoming.accident) {

// Accident

digitalWrite(RELAY, HIGH);

digitalWrite(RED, HIGH);

digitalWrite(BLUE, LOW);

digitalWrite(BUZZER, HIGH);

Serial.println("ACCIDENT - BIKE OFF");

if (!alertSent) {

Blynk.logEvent("accident_alert", "Accident Detected!");

sendTelegram("Accident Detected!");

alertSent = true;

}

}

else if (incoming.alcohol) {

// Alcohol detected

digitalWrite(RELAY, HIGH);

digitalWrite(RED, millis()%400 < 200); // blink

digitalWrite(BLUE, LOW);

digitalWrite(BUZZER, LOW);

Serial.println("ALCOHOL DETECTED - BIKE OFF");

alertSent = false;

}

else {

// Safe

digitalWrite(RELAY, LOW);

digitalWrite(RED, LOW);

digitalWrite(BLUE, HIGH);

digitalWrite(BUZZER, LOW);

Serial.println("SAFE - BIKE ON");

alertSent = false;

}

Serial.println("========================");

delay(500);

}